NoctoPark Station

# Introduction

NoctoPark parking payment station makes parking payments quick and hassle-free. Packed with advanced features and powered by our proprietary parking management software, it’s designed to deliver a seamless experience for both users and operators.

# Key Features:

- Ergonomic and intuitive operation for enhanced customer experience

- 15-inch color touchscreen with guided instructions and customizable display

- Illuminated button for ticket dispensing

- Bilingual display interface (English and Arabic configurable)

- QR code printer for ticket printing (LPR Backup)

- Built to withstand harsh weather conditions for long-term reliability

# Typical Application

Entrance/Exit

Barrier Gate

Payment Station

Server Room/Management

# Specification

| General |  |
| --- | --- |
| Power Consumption | 750W (with heater off)/1050W (with heater on) |
| Power Supply | 120VAC |
| Dimension | 17.5*16*51.5 inch (W*D*H) |
| Working Temperature | -20°C to 60°C |
| Housing | Weather-proof epoxy paining |
| Protection | Waterproof and dust resistant (Frame) |
| Industrial Lock | Cam lock |
| Touch Screen |  |
| Ever Glory TFT LCD Monitor ZD-150MT3C-V23 |  |
| Resolution | 1024(H)×768(V) |
| Dimension | 15 inch |
| Angle | 80/80/80/80 (L/R/U/D) |
| Color | 16.7M／262K |
| Brightness | 950cd/m2 |
| Contrast Ratio | 600:1 |
| Backlight | LED |
| Display Size | 304.128(H)×228.096(V)mm |
| Response Time | 8ms |
| Transmittance | >95% |
| Interface | VGA, HDMI, DC, PCAUDIO |
| Surface Hardness | 7H |
| IP Ingress Protection | IP65 (front panel) |
| Code Scanner |  |
| DATALOGIC Code Scanner |  |
| Light Source | 650 nm visible laser diode |
| Pitch | ±40 degree |
| Roll | ±180 degree |
| Yaw | ±40 degree |
| Decoding Capability | 1D / Linear Codes: Autodiscriminates all standard 1D codes including GS1 Data Bar linear codes 2D Codes: - Aztec Code - China Han Xin Code - Data Matrix - Maxi Code - Micro QR Code - QR Code Postal Codes: - Australian Post - British Post - China Post - IMB - Japanese Post - KIX Post - Korea Post - Planet Code - Postnet - Royal Mail Code (RM4SCC) Stacked Codes: - EAN/JAN Composites - GS1 DataBar Composites - GS1 DataBar Expanded Stacked - GS1 DataBar Stacked - GS1 DataBar Stacked Omnidirectional - MacroPDF - MicroPDF417 - PDF417 - UPC A/E Composites |
| Features | Highly visible 4-Dot aimer with center-cross for targeted scanning Omnidirectional reading Advanced motion tolerance optics Image capture and document scanning capabilities Reads bar codes down to 4mils Reads 1D, 2D and postal codes plus stacked and composite codes Datalogic's Green Spot for good-read feedback Integrated good-read beeper High-impact resin creates a solvent-tolerant enclosure |
| IP Ingress Protection | IP54 |
| Receipt Printer |  |
| Custom VKP80III Receipt Printer |  |
| Flash | 4 megabyte |
| RAM | 16 kilobyte |
| Interface | Serial, USB |
| Max B/W Print Resolution | 203 dpi |
| Max H-Document Size | 3.25 in |
| Max Media Weight | 110 gram per square metre |
| Max Mono Print Speed | 519.7 Inches per minute |
| Max Print Size | Roll (3.25 in) |
| Max Roll Diameter | 11.8 inch |
| Min H-Document Size | 2.35 in |
| Min Media Weight | 55 gram per square metre |
| Print Output Type | Monochrome |
| Print Speed | Up to 519.7 inch/min - max speed |
| Barcode Symbology Supported | PDF417, QR code |
| Media Feeders | 1 x manual - 1 roll - Roll (3.25 in) |
| Media Sizes | Roll (3.25 in) |
| Auto Paper Cutter | Yes |
| Media Type | Thermal paper |
| Mini PC |  |
| MeLE Mini PC |  |
| CPU | Intel Celeron 11th Gen Jasper Lake N5105, Quad cores, Frequency 2.00 GHz-2.90 GHz |
| GPU | Intel UHD Graphics(Graphics Frequency 450 MHz-800MHz) |
| Operating System | OS-P |
| Memory | LPDDR4 8GB |
| Storage | 256GB eMMC On board |
| Wireless Connectivity | Bluetooth 5.1, WiFi (802.11ac) 2.4GHz/5GHz |
| Ethernet | 1000Mbps LAN |
| Video Output | 2x HDMI® 2.0 (4K@60Hz), 1x USB Type-C (4K@60Hz) |
| Audio Output | 3.5mm Audio Combo Jack |
| USB | 3x USB 3.0, 1x USB Type-C |
| Modicon |  |
| Schneider Electric Modicon |  |
| Range of Product | Modicon M221 |
| Discrete Input Number | 9, discrete input IEC 61131-2 Type 1 |
| Analogue Input Number | 2 0...10 V |
| Discrete Output Type | Relay normally open |
| Discrete Output Number | 7 relay |
| Discrete Output Voltage | 5...125 V DC 5...250 V AC |
| Discrete Output Current | 2 A |
| Discrete I/O Number | 16 |
| Maximum Number of I/O Expansion Module | 4 local 11 remote |
| Supply Voltage Limits | 85…264 V |
| Network Frequency | 50/60 Hz |
| Inrush Current | 40 A |
| Maximum Power Consumption in VA | 49 VA 100...240 V with max number of I/O expansion module 33 VA 100...240 V without I/O expansion module |
| Power Supply Output Current | 0.325 A 5 V expansion bus 0.12 A 24 V expansion bus |
| Discrete Input Logic | Sink or source (positive/negative) |
| Discrete Input Voltage | 24 V |
| Discrete Input voltage type | DC |
| Analogue Input Resolution | 10 bits |
| LSB Value | 10 mV |
| Conversion Time | 1 ms per channel + 1 controller cycle time analog input |
| Permitted Overload on Inputs | +/- 30 V DC 5 min maximum)analog input +/- 13 V DC permanent)analog input |
| Voltage State 1 Guaranteed | >= 15 V input |
| Voltage State 0 Guaranteed | <= 5 V input |
| Discrete Input Current | 7 mA discrete input 5 mA fast input |
| Input Impedance | 3.4 kOhm discrete input 100 kOhm analog input 4.9 kOhm fast input |
| Response Time | 35 µs turn-off, I2...I5 input 10 ms turn-on output 10 ms turn-off output 5 µs turn-on, I0, I1, I6, I7 fast input 35 µs turn-on, other terminals input 5 µs turn-off, I0, I1, I6, I7 fast input 100 µs turn-off, other terminals input |
| Configurable Filtering Time | 0 ms input 3 ms input 12 ms input |
| Output Voltage Limits | 125 V DC 277 V AC |
| Maximum Current per Output Common | 6 A COM 1 7 A COM 0 |
| Absolute Accuracy Error | +/- 1 % of full scale analog input |
| Electrical Durability | 100000 cycles AC-12, 120 V, 240 VA, resistive 100000 cycles AC-12, 240 V, 480 VA, resistive 300000 cycles AC-12, 120 V, 80 VA, resistive 300000 cycles AC-12, 240 V, 160 VA, resistive 100000 cycles AC-15, cos phi = 0.35, 120 V, 60 VA, inductive 100000 cycles AC-15, cos phi = 0.35, 240 V, 120 VA, inductive 300000 cycles AC-15, cos phi = 0.35, 120 V, 18 VA, inductive 300000 cycles AC-15, cos phi = 0.35, 240 V, 36 VA, inductive 100000 cycles AC-14, cos phi = 0.7, 120 V, 120 VA, inductive 100000 cycles AC-14, cos phi = 0.7, 240 V, 240 VA, inductive 300000 cycles AC-14, cos phi = 0.7, 120 V, 36 VA, inductive 300000 cycles AC-14, cos phi = 0.7, 240 V, 72 VA, inductive 100000 cycles DC-12, 24 V, 48 W, resistive 300000 cycles DC-12, 24 V, 16 W, resistive 100000 cycles DC-13, 24 V, 24 W, inductive (L/R = 7 ms) 300000 cycles DC-13, 24 V, 7.2 W, inductive (L/R = 7 ms) |
| Switching Frequency | 20 switching operations/minute with maximum load |
| Mechanical Durability | 20000000 cycles relay output |
| Minimum Load | 1 mA 5 V DC relay output |
| Protection Type | Without protection 5 A |
| Reset Time | 1 s |
| Memory Capacity | 256 kB user application and data RAM 10000 instructions 256 kB internal variables RAM |
| Data Backed Up | 256 kB built-in flash memory backup of application and data |
| Battery Type | BR2032 or CR2032X lithium non-rechargeable |
| Backup Time | 1 year 77 °F (25 °C) by interruption of power supply) |
| Execution Time for 1 KInstruction | 0.3 ms event and periodic task |
| Execution Time per Instruction | 0.2 µs Boolean |
| Exct Time for Event Task | 60 µs response time |
| Maximum Size of Object Areas | 255 %C counters 512 %KW constant words 255 %TM timers 512 %M memory bits 8000 %MW memory words |
| Realtime Clock | With |
| Clock Drift | <= 30 s/month 77 °F (25 °C) |
| Regulation Loop | Adjustable PID regulator up to 14 simultaneous loops |
| Counting Input Number | 4 fast input (HSC mode) 100 kHz 32 bits |
| Counter Function | Pulse/direction A/B Single phase |
| Integrated Connection Type | USB port mini B USB 2.0 Non isolated serial link serial 1 RJ45 RS232/RS485 Ethernet RJ45 |
| Supply | Serial)serial link supply 5 V, <200 mA |
| Transmission Rate | 1.2...115.2 kbit/s (115.2 kbit/s by default) 49.2 ft (15 m) RS485 1.2...115.2 kbit/s (115.2 kbit/s by default) 9.8 ft (3 m) RS232 480 Mbit/s USB |
| Communication Port Protocol | USB port USB - SoMachine-Network Non isolated serial link Modbus master/slave - RTU/ASCII or SoMachine-Network Ethernet |
| Port Ethernet | 10BASE-T/100BASE-TX 1 328.08 ft (100 m) copper cable |
| Communication Service | DHCP client Ethernet/IP adapter Modbus TCP server Modbus TCP slave device Modbus TCP client |
| Insulation | Between input and internal logic 500 V AC Non-insulated between analogue input and internal logic Non-insulated between analogue inputs Between supply and ground 1500 V AC Between sensor power supply and ground 500 V AC Between input and ground 500 V AC Between output and ground 1500 V AC Between supply and internal logic 2300 V AC Between sensor power supply and internal logic 500 V AC Between output and internal logic 2300 V AC Between Ethernet terminal and internal logic 500 V AC Between supply and sensor power supply 2300 V AC |
| Marking | CE |
| Standards | IEC 61131-2 UL 508 CAN/CSA C22.2 No. 213 IACS E10 ANSI/ISA 12-12-01 |
| Certifications | cULus LR RCM EAC ABS DNV-GL CE UKCA cULus HazLoc |
| Resistance to Electrostatic Discharge | 8 kV in air IEC 61000-4-2 4 kV on contact IEC 61000-4-2 |
| Resistance to Magnetic Fields | 98.4 A/ft (30 A/m) 50/60 Hz IEC 61000-4-8 |
| Resistance to Fast Transients | 2 kV IEC 61000-4-4 power lines) 2 kV IEC 61000-4-4 relay output) 1 kV IEC 61000-4-4 I/O) 1 kV IEC 61000-4-4 Ethernet line) 1 kV IEC 61000-4-4 serial link) |
| Surge Withstand | 2 kV power lines (AC) common mode IEC 61000-4-5 2 kV relay output common mode IEC 61000-4-5 1 kV I/O common mode IEC 61000-4-5 1 kV shielded cable common mode IEC 61000-4-5 0.5 kV power lines (DC) differential mode IEC 61000-4-5 1 kV power lines (AC) differential mode IEC 61000-4-5 1 kV relay output differential mode IEC 61000-4-5 0.5 kV power lines (DC) common mode IEC 61000-4-5 |
| Resistance to Conducted Disturbances | 10 V 0.15...80 MHz IEC 61000-4-6 3 V 0.1...80 MHz Marine specification (LR, ABS, DNV, GL) 10 V spot frequency (2, 3, 4, 6.2, 8.2, 12.6, 16.5, 18.8, 22, 25 MHz) Marine specification (LR, ABS, DNV, GL) |
| Electromagnetic Emission | Conducted emissions 79 dBμV/m QP/66 dBμV/m AV power lines (AC))0.15…0.5 MHz IEC 55011 Conducted emissions 73 dBμV/m QP/60 dBμV/m AV power lines (AC))0.5…300 MHz IEC 55011 Conducted emissions 120...69 dBµV/m QP power lines)10…150 kHz IEC 55011 Conducted emissions 63 dBμV/m QP power lines)1.5…30 MHz IEC 55011 Radiated emissions 40 dBμV/m QP class A 10 m)30…230 MHz IEC 55011 Conducted emissions 79...63 dBμV/m QP power lines)150…1500 kHz IEC 55011 Radiated emissions 47 dBμV/m QP class A 10 m)200…1000 MHz IEC 55011 |
| Immunity to Microbreaks | 10 ms |
| IP Degree of Protection | IP20 with protective cover in place |
| Network Switch |  |
| Hikvision Smart Network Switch |  |
| Ports | 8 × Gigabit PoE port,1 × Gigabit RJ45 port,1 × Gigabit fiber optical port |
| MAC Address Table | 4 K |
| Switching Capacity | 20 Gbps |
| Packet Forwarding Rate | 14.88 Mpps |
| Internal Cache | 1.5 Mbits |
| PoE Standard | IEEE 802.3af,IEEE 802.3at |
| PoE Port | PoE: Ports 1 to 8 |
| Max. Port Power | 30 W |
| EMC | FCC (47 CFR Part 15, Subpart B),CE-EMC (EN 55032: 2015+A11: 2020, EN IEC 61000-3-2: 2019, EN 61000-3-3: 2013+A1: 2019, EN 50130-4: 2011+A1: 2014, EN 55035: 2017+A11: 2020),IC (ICES-003: Issue 7:2020),RCM (AS/NZS CISPR 32: 2015) |
| Safety | UL (UL 60950-1),CB (AMD1:2009, AMD2:2013, IEC 62368-1: 2014 (Second Edition), CE-LVD (EN 62368-1: 2014+A11: 2017) |
| USB Hub |  |
| StarTech USB Hub with Power |  |
| Hardware Interface | USB 2.0 |
| Additional Features | Powered |
| Total Usb Ports | 7 |
| Data Transfer Rate | 480 Megabits Per Second |
| Operating System | OS Independent; No additional drivers or software required |
| Colour | Black |
| Enclosure Material | Plastic |
| Item Dimensions L x W x H | 9.9L x 6.1W x 2H Centimetres |
| Item Weight | 0.17 Pounds |

# Dimension